DMZ mit zwei ASA 5506 (funktional, testbar)

# 1. Netzstruktur

Die DMZ befindet sich zwischen zwei ASA-Firewalls:
- ASA1 trennt das externe Netzwerk (Internet) von der DMZ.
- ASA2 trennt die DMZ vom internen Netzwerk.

- Ein Switch wird benötigt um den Server mit beiden Asa Firewalls zu verbinden.

# 2. IP-Plan

| Zone / Gerät | IP-Adresse | Gateway |
| --- | --- | --- |
| Extern (PC0) | 192.168.10.100 | 192.168.10.1 |
| ASA1 G0/0 (outside) | 192.168.10.1 | - |
| ASA1 G0/1 (dmz) | 192.168.20.1 | - |
| DMZ Server | 192.168.20.10 | 192.168.20.1 |
| ASA2 G0/0 (dmz) | 192.168.20.2 | - |
| ASA2 G0/1 (inside) | 192.168.30.1 | - |
| Intern (PC1) | 192.168.30.100 | 192.168.30.1 |

# 3. ASA1-Konfiguration (Outside ↔ DMZ)

interface g0/0
 nameif outside
 security-level 0
 ip address 192.168.10.1 255.255.255.0
 no shutdown

interface g0/1
 nameif dmz
 security-level 50
 ip address 192.168.20.1 255.255.255.0
 no shutdown

object network DMZ-WEB
 host 192.168.20.10
 nat (dmz,outside) static 192.168.10.200

access-list OUTSIDE_TO_DMZ permit tcp any host 192.168.20.10 eq 80
access-group OUTSIDE_TO_DMZ in interface outside

# 4. ASA2-Konfiguration (DMZ ↔ Inside)

interface g0/0
 nameif dmz
 security-level 50
 ip address 192.168.20.2 255.255.255.0
 no shutdown

interface g0/1
 nameif inside
 security-level 100
 ip address 192.168.30.1 255.255.255.0
 no shutdown

access-list INSIDE_ALLOW permit ip any any
access-group INSIDE_ALLOW in interface inside

# 5. Testfälle

Diese Tests sollten durchgeführt werden:

| Quelle → Ziel | Protokoll | Erwartung |
| --- | --- | --- |
| PC0 → 192.168.10.200 | HTTP | erlaubt |
| PC0 → PC1 | Ping | verboten |
| PC1 → 192.168.20.10 | HTTP | verboten |
| Server → PC1 | Ping | verboten |